EXPERIMENT :2

## Aim

To simulate and analyze the performance of microwave transmission lines and two-port networks using MATLAB and Scattering Parameters (S-parameters).

## Objectives

- To study the characteristics of microwave transmission lines and two-port networks.

- To calculate and analyze S-parameters such as S11, S21, S12, and S22.

- To evaluate reflection coefficient, transmission coefficient, and return loss.

- To simulate and visualize the network performance using MATLAB.

PROGRAM CODE:

clc; clear; close all;

% Input parameters

f = 8:0.05:12;              % Frequency in GHz

f0 = 10;                    % Center frequency

Z0 = 50;                    % Reference impedance (ohms)

L = 50e-3;                  % Length in metres

% Illustrative two-port model

S11dB = -25 - 15*(f-10).^2;

S21dB = -2 - 0.1*(f-10).^2;

% Linear S-parameter magnitudes

S11 = 10.^(S11dB/20);

S21 = 10.^(S21dB/20);

% Performance calculations

RL = -20*log10(abs(S11));   % Return loss (dB)

IL = -20*log10(abs(S21));   % Insertion loss (dB)

Eff = abs(S21).^2*100;      % Transmission efficiency (%)

% Find values at 10 GHz

[~,i0] = min(abs(f-f0));

% Find optimum frequency

[~,ib] = min(abs(S11));

[~,it] = max(abs(S21));

% Display values in Command Window

fprintf('\n MICROWAVE TWO-PORT SIMULATION\n');

fprintf('-----------------------------------\n');

fprintf('S11 at 10 GHz             = %.3f dB\n',S11dB(i0));

fprintf('S21 at 10 GHz             = %.3f dB\n',S21dB(i0));

fprintf('Return Loss               = %.3f dB\n',RL(i0));

fprintf('Insertion Loss            = %.3f dB\n',IL(i0));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(i0));

fprintf('Best Operating Frequency  = %.2f GHz\n',f(ib));

fprintf('Maximum S21 Frequency     = %.2f GHz\n',f(it));

fprintf('\n VALUES AT BEST FREQUENCY\n');

fprintf('S11                       = %.3f dB\n',S11dB(ib));

fprintf('S21                       = %.3f dB\n',S21dB(ib));

fprintf('Return Loss               = %.3f dB\n',RL(ib));

fprintf('Insertion Loss            = %.3f dB\n',IL(ib));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(ib));

% Plot all six parameters

figure('Color','w');

subplot(2,3,1);

plot(f,S11dB,'b','LineWidth',1.5);

title('S11'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,2);

plot(f,S21dB,'r','LineWidth',1.5);

title('S21'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,3);

plot(f,RL,'m','LineWidth',1.5);

title('Return Loss'); xlabel('Frequency (GHz)');

ylabel('RL (dB)'); grid on;

subplot(2,3,4);

plot(f,IL,'k','LineWidth',1.5);

title('Insertion Loss'); xlabel('Frequency (GHz)');

ylabel('IL (dB)'); grid on;

subplot(2,3,5);

plot(f,Eff,'g','LineWidth',1.5);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)'); grid on;

subplot(2,3,6);

plot(f,S11dB,'b','LineWidth',1.5); hold on;

plot(f,S21dB,'r','LineWidth',1.5);

xline(f(ib),'k--','Best frequency');

title('Optimum Frequency');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('S11','S21','Location','best');

grid on;

sgtitle('Microwave Two-Port Network (8-12 GHz)');clc; clear; close all;

% Input parameters

f = 8:0.05:12;              % Frequency in GHz

f0 = 10;                    % Center frequency

Z0 = 50;                    % Reference impedance (ohms)

L = 50e-3;                  % Length in metres

% Illustrative two-port model

S11dB = -25 - 15*(f-10).^2;

S21dB = -2 - 0.1*(f-10).^2;

% Linear S-parameter magnitudes

S11 = 10.^(S11dB/20);

S21 = 10.^(S21dB/20);

% Performance calculations

RL = -20*log10(abs(S11));   % Return loss (dB)

IL = -20*log10(abs(S21));   % Insertion loss (dB)

Eff = abs(S21).^2*100;      % Transmission efficiency (%)

% Find values at 10 GHz

[~,i0] = min(abs(f-f0));

% Find optimum frequency

[~,ib] = min(abs(S11));

[~,it] = max(abs(S21));

% Display values in Command Window

fprintf('\n MICROWAVE TWO-PORT SIMULATION\n');

fprintf('-----------------------------------\n');

fprintf('S11 at 10 GHz             = %.3f dB\n',S11dB(i0));

fprintf('S21 at 10 GHz             = %.3f dB\n',S21dB(i0));

fprintf('Return Loss               = %.3f dB\n',RL(i0));

fprintf('Insertion Loss            = %.3f dB\n',IL(i0));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(i0));

fprintf('Best Operating Frequency  = %.2f GHz\n',f(ib));

fprintf('Maximum S21 Frequency     = %.2f GHz\n',f(it));

fprintf('\n VALUES AT BEST FREQUENCY\n');

fprintf('S11                       = %.3f dB\n',S11dB(ib));

fprintf('S21                       = %.3f dB\n',S21dB(ib));

fprintf('Return Loss               = %.3f dB\n',RL(ib));

fprintf('Insertion Loss            = %.3f dB\n',IL(ib));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(ib));

% Plot all six parameters

figure('Color','w');

subplot(2,3,1);

plot(f,S11dB,'b','LineWidth',1.5);

title('S11'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,2);

plot(f,S21dB,'r','LineWidth',1.5);

title('S21'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,3);

plot(f,RL,'m','LineWidth',1.5);

title('Return Loss'); xlabel('Frequency (GHz)');

ylabel('RL (dB)'); grid on;

subplot(2,3,4);

plot(f,IL,'k','LineWidth',1.5);

title('Insertion Loss'); xlabel('Frequency (GHz)');

ylabel('IL (dB)'); grid on;

subplot(2,3,5);

plot(f,Eff,'g','LineWidth',1.5);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)'); grid on;

subplot(2,3,6);

plot(f,S11dB,'b','LineWidth',1.5); hold on;

plot(f,S21dB,'r','LineWidth',1.5);

xline(f(ib),'k--','Best frequency');

title('Optimum Frequency');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('S11','S21','Location','best');

grid on;

sgtitle('Microwave Two-Port Network (8-12 GHz)');clc; clear; close all;

% Input parameters

f = 8:0.05:12;              % Frequency in GHz

f0 = 10;                    % Center frequency

Z0 = 50;                    % Reference impedance (ohms)

L = 50e-3;                  % Length in metres

% Illustrative two-port model

S11dB = -25 - 15*(f-10).^2;

S21dB = -2 - 0.1*(f-10).^2;

% Linear S-parameter magnitudes

S11 = 10.^(S11dB/20);

S21 = 10.^(S21dB/20);

% Performance calculations

RL = -20*log10(abs(S11));   % Return loss (dB)

IL = -20*log10(abs(S21));   % Insertion loss (dB)

Eff = abs(S21).^2*100;      % Transmission efficiency (%)

% Find values at 10 GHz

[~,i0] = min(abs(f-f0));

% Find optimum frequency

[~,ib] = min(abs(S11));

[~,it] = max(abs(S21));

% Display values in Command Window

fprintf('\n MICROWAVE TWO-PORT SIMULATION\n');

fprintf('-----------------------------------\n');

fprintf('S11 at 10 GHz             = %.3f dB\n',S11dB(i0));

fprintf('S21 at 10 GHz             = %.3f dB\n',S21dB(i0));

fprintf('Return Loss               = %.3f dB\n',RL(i0));

fprintf('Insertion Loss            = %.3f dB\n',IL(i0));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(i0));

fprintf('Best Operating Frequency  = %.2f GHz\n',f(ib));

fprintf('Maximum S21 Frequency     = %.2f GHz\n',f(it));

fprintf('\n VALUES AT BEST FREQUENCY\n');

fprintf('S11                       = %.3f dB\n',S11dB(ib));

fprintf('S21                       = %.3f dB\n',S21dB(ib));

fprintf('Return Loss               = %.3f dB\n',RL(ib));

fprintf('Insertion Loss            = %.3f dB\n',IL(ib));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(ib));

% Plot all six parameters

figure('Color','w');

subplot(2,3,1);

plot(f,S11dB,'b','LineWidth',1.5);

title('S11'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,2);

plot(f,S21dB,'r','LineWidth',1.5);

title('S21'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,3);

plot(f,RL,'m','LineWidth',1.5);

title('Return Loss'); xlabel('Frequency (GHz)');

ylabel('RL (dB)'); grid on;

subplot(2,3,4);

plot(f,IL,'k','LineWidth',1.5);

title('Insertion Loss'); xlabel('Frequency (GHz)');

ylabel('IL (dB)'); grid on;

subplot(2,3,5);

plot(f,Eff,'g','LineWidth',1.5);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)'); grid on;

subplot(2,3,6);

plot(f,S11dB,'b','LineWidth',1.5); hold on;

plot(f,S21dB,'r','LineWidth',1.5);

xline(f(ib),'k--','Best frequency');

title('Optimum Frequency');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('S11','S21','Location','best');

grid on;

sgtitle('Microwave Two-Port Network (8-12 GHz)');clc; clear; close all;

% Input parameters

f = 8:0.05:12;              % Frequency in GHz

f0 = 10;                    % Center frequency

Z0 = 50;                    % Reference impedance (ohms)

L = 50e-3;                  % Length in metres

% Illustrative two-port model

S11dB = -25 - 15*(f-10).^2;

S21dB = -2 - 0.1*(f-10).^2;

% Linear S-parameter magnitudes

S11 = 10.^(S11dB/20);

S21 = 10.^(S21dB/20);

% Performance calculations

RL = -20*log10(abs(S11));   % Return loss (dB)

IL = -20*log10(abs(S21));   % Insertion loss (dB)

Eff = abs(S21).^2*100;      % Transmission efficiency (%)

% Find values at 10 GHz

[~,i0] = min(abs(f-f0));

% Find optimum frequency

[~,ib] = min(abs(S11));

[~,it] = max(abs(S21));

% Display values in Command Window

fprintf('\n MICROWAVE TWO-PORT SIMULATION\n');

fprintf('-----------------------------------\n');

fprintf('S11 at 10 GHz             = %.3f dB\n',S11dB(i0));

fprintf('S21 at 10 GHz             = %.3f dB\n',S21dB(i0));

fprintf('Return Loss               = %.3f dB\n',RL(i0));

fprintf('Insertion Loss            = %.3f dB\n',IL(i0));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(i0));

fprintf('Best Operating Frequency  = %.2f GHz\n',f(ib));

fprintf('Maximum S21 Frequency     = %.2f GHz\n',f(it));

fprintf('\n VALUES AT BEST FREQUENCY\n');

fprintf('S11                       = %.3f dB\n',S11dB(ib));

fprintf('S21                       = %.3f dB\n',S21dB(ib));

fprintf('Return Loss               = %.3f dB\n',RL(ib));

fprintf('Insertion Loss            = %.3f dB\n',IL(ib));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(ib));

% Plot all six parameters

figure('Color','w');

subplot(2,3,1);

plot(f,S11dB,'b','LineWidth',1.5);

title('S11'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,2);

plot(f,S21dB,'r','LineWidth',1.5);

title('S21'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,3);

plot(f,RL,'m','LineWidth',1.5);

title('Return Loss'); xlabel('Frequency (GHz)');

ylabel('RL (dB)'); grid on;

subplot(2,3,4);

plot(f,IL,'k','LineWidth',1.5);

title('Insertion Loss'); xlabel('Frequency (GHz)');

ylabel('IL (dB)'); grid on;

subplot(2,3,5);

plot(f,Eff,'g','LineWidth',1.5);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)'); grid on;

subplot(2,3,6);

plot(f,S11dB,'b','LineWidth',1.5); hold on;

plot(f,S21dB,'r','LineWidth',1.5);

xline(f(ib),'k--','Best frequency');

title('Optimum Frequency');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('S11','S21','Location','best');

grid on;

sgtitle('Microwave Two-Port Network (8-12 GHz)');clc; clear; close all;

% Input parameters

f = 8:0.05:12;              % Frequency in GHz

f0 = 10;                    % Center frequency

Z0 = 50;                    % Reference impedance (ohms)

L = 50e-3;                  % Length in metres

% Illustrative two-port model

S11dB = -25 - 15*(f-10).^2;

S21dB = -2 - 0.1*(f-10).^2;

% Linear S-parameter magnitudes

S11 = 10.^(S11dB/20);

S21 = 10.^(S21dB/20);

% Performance calculations

RL = -20*log10(abs(S11));   % Return loss (dB)

IL = -20*log10(abs(S21));   % Insertion loss (dB)

Eff = abs(S21).^2*100;      % Transmission efficiency (%)

% Find values at 10 GHz

[~,i0] = min(abs(f-f0));

% Find optimum frequency

[~,ib] = min(abs(S11));

[~,it] = max(abs(S21));

% Display values in Command Window

fprintf('\n MICROWAVE TWO-PORT SIMULATION\n');

fprintf('-----------------------------------\n');

fprintf('S11 at 10 GHz             = %.3f dB\n',S11dB(i0));

fprintf('S21 at 10 GHz             = %.3f dB\n',S21dB(i0));

fprintf('Return Loss               = %.3f dB\n',RL(i0));

fprintf('Insertion Loss            = %.3f dB\n',IL(i0));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(i0));

fprintf('Best Operating Frequency  = %.2f GHz\n',f(ib));

fprintf('Maximum S21 Frequency     = %.2f GHz\n',f(it));

fprintf('\n VALUES AT BEST FREQUENCY\n');

fprintf('S11                       = %.3f dB\n',S11dB(ib));

fprintf('S21                       = %.3f dB\n',S21dB(ib));

fprintf('Return Loss               = %.3f dB\n',RL(ib));

fprintf('Insertion Loss            = %.3f dB\n',IL(ib));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(ib));

% Plot all six parameters

figure('Color','w');

subplot(2,3,1);

plot(f,S11dB,'b','LineWidth',1.5);

title('S11'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,2);

plot(f,S21dB,'r','LineWidth',1.5);

title('S21'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,3);

plot(f,RL,'m','LineWidth',1.5);

title('Return Loss'); xlabel('Frequency (GHz)');

ylabel('RL (dB)'); grid on;

subplot(2,3,4);

plot(f,IL,'k','LineWidth',1.5);

title('Insertion Loss'); xlabel('Frequency (GHz)');

ylabel('IL (dB)'); grid on;

subplot(2,3,5);

plot(f,Eff,'g','LineWidth',1.5);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)'); grid on;

subplot(2,3,6);

plot(f,S11dB,'b','LineWidth',1.5); hold on;

plot(f,S21dB,'r','LineWidth',1.5);

xline(f(ib),'k--','Best frequency');

title('Optimum Frequency');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('S11','S21','Location','best');

grid on;

sgtitle('Microwave Two-Port Network (8-12 GHz)');clc; clear; close all;

% Input parameters

f = 8:0.05:12;              % Frequency in GHz

f0 = 10;                    % Center frequency

Z0 = 50;                    % Reference impedance (ohms)

L = 50e-3;                  % Length in metres

% Illustrative two-port model

S11dB = -25 - 15*(f-10).^2;

S21dB = -2 - 0.1*(f-10).^2;

% Linear S-parameter magnitudes

S11 = 10.^(S11dB/20);

S21 = 10.^(S21dB/20);

% Performance calculations

RL = -20*log10(abs(S11));   % Return loss (dB)

IL = -20*log10(abs(S21));   % Insertion loss (dB)

Eff = abs(S21).^2*100;      % Transmission efficiency (%)

% Find values at 10 GHz

[~,i0] = min(abs(f-f0));

% Find optimum frequency

[~,ib] = min(abs(S11));

[~,it] = max(abs(S21));

% Display values in Command Window

fprintf('\n MICROWAVE TWO-PORT SIMULATION\n');

fprintf('-----------------------------------\n');

fprintf('S11 at 10 GHz             = %.3f dB\n',S11dB(i0));

fprintf('S21 at 10 GHz             = %.3f dB\n',S21dB(i0));

fprintf('Return Loss               = %.3f dB\n',RL(i0));

fprintf('Insertion Loss            = %.3f dB\n',IL(i0));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(i0));

fprintf('Best Operating Frequency  = %.2f GHz\n',f(ib));

fprintf('Maximum S21 Frequency     = %.2f GHz\n',f(it));

fprintf('\n VALUES AT BEST FREQUENCY\n');

fprintf('S11                       = %.3f dB\n',S11dB(ib));

fprintf('S21                       = %.3f dB\n',S21dB(ib));

fprintf('Return Loss               = %.3f dB\n',RL(ib));

fprintf('Insertion Loss            = %.3f dB\n',IL(ib));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(ib));

% Plot all six parameters

figure('Color','w');

subplot(2,3,1);

plot(f,S11dB,'b','LineWidth',1.5);

title('S11'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,2);

plot(f,S21dB,'r','LineWidth',1.5);

title('S21'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,3);

plot(f,RL,'m','LineWidth',1.5);

title('Return Loss'); xlabel('Frequency (GHz)');

ylabel('RL (dB)'); grid on;

subplot(2,3,4);

plot(f,IL,'k','LineWidth',1.5);

title('Insertion Loss'); xlabel('Frequency (GHz)');

ylabel('IL (dB)'); grid on;

subplot(2,3,5);

plot(f,Eff,'g','LineWidth',1.5);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)'); grid on;

subplot(2,3,6);

plot(f,S11dB,'b','LineWidth',1.5); hold on;

plot(f,S21dB,'r','LineWidth',1.5);

xline(f(ib),'k--','Best frequency');

title('Optimum Frequency');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('S11','S21','Location','best');

grid on;

sgtitle('Microwave Two-Port Network (8-12 GHz)');clc; clear; close all;

% Input parameters

f = 8:0.05:12;              % Frequency in GHz

f0 = 10;                    % Center frequency

Z0 = 50;                    % Reference impedance (ohms)

L = 50e-3;                  % Length in metres

% Illustrative two-port model

S11dB = -25 - 15*(f-10).^2;

S21dB = -2 - 0.1*(f-10).^2;

% Linear S-parameter magnitudes

S11 = 10.^(S11dB/20);

S21 = 10.^(S21dB/20);

% Performance calculations

RL = -20*log10(abs(S11));   % Return loss (dB)

IL = -20*log10(abs(S21));   % Insertion loss (dB)

Eff = abs(S21).^2*100;      % Transmission efficiency (%)

% Find values at 10 GHz

[~,i0] = min(abs(f-f0));

% Find optimum frequency

[~,ib] = min(abs(S11));

[~,it] = max(abs(S21));

% Display values in Command Window

fprintf('\n MICROWAVE TWO-PORT SIMULATION\n');

fprintf('-----------------------------------\n');

fprintf('S11 at 10 GHz             = %.3f dB\n',S11dB(i0));

fprintf('S21 at 10 GHz             = %.3f dB\n',S21dB(i0));

fprintf('Return Loss               = %.3f dB\n',RL(i0));

fprintf('Insertion Loss            = %.3f dB\n',IL(i0));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(i0));

fprintf('Best Operating Frequency  = %.2f GHz\n',f(ib));

fprintf('Maximum S21 Frequency     = %.2f GHz\n',f(it));

fprintf('\n VALUES AT BEST FREQUENCY\n');

fprintf('S11                       = %.3f dB\n',S11dB(ib));

fprintf('S21                       = %.3f dB\n',S21dB(ib));

fprintf('Return Loss               = %.3f dB\n',RL(ib));

fprintf('Insertion Loss            = %.3f dB\n',IL(ib));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(ib));

% Plot all six parameters

figure('Color','w');

subplot(2,3,1);

plot(f,S11dB,'b','LineWidth',1.5);

title('S11'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,2);

plot(f,S21dB,'r','LineWidth',1.5);

title('S21'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,3);

plot(f,RL,'m','LineWidth',1.5);

title('Return Loss'); xlabel('Frequency (GHz)');

ylabel('RL (dB)'); grid on;

subplot(2,3,4);

plot(f,IL,'k','LineWidth',1.5);

title('Insertion Loss'); xlabel('Frequency (GHz)');

ylabel('IL (dB)'); grid on;

subplot(2,3,5);

plot(f,Eff,'g','LineWidth',1.5);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)'); grid on;

subplot(2,3,6);

plot(f,S11dB,'b','LineWidth',1.5); hold on;

plot(f,S21dB,'r','LineWidth',1.5);

xline(f(ib),'k--','Best frequency');

title('Optimum Frequency');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('S11','S21','Location','best');

grid on;

sgtitle('Microwave Two-Port Network (8-12 GHz)');clc; clear; close all;

% Input parameters

f = 8:0.05:12;              % Frequency in GHz

f0 = 10;                    % Center frequency

Z0 = 50;                    % Reference impedance (ohms)

L = 50e-3;                  % Length in metres

% Illustrative two-port model

S11dB = -25 - 15*(f-10).^2;

S21dB = -2 - 0.1*(f-10).^2;

% Linear S-parameter magnitudes

S11 = 10.^(S11dB/20);

S21 = 10.^(S21dB/20);

% Performance calculations

RL = -20*log10(abs(S11));   % Return loss (dB)

IL = -20*log10(abs(S21));   % Insertion loss (dB)

Eff = abs(S21).^2*100;      % Transmission efficiency (%)

% Find values at 10 GHz

[~,i0] = min(abs(f-f0));

% Find optimum frequency

[~,ib] = min(abs(S11));

[~,it] = max(abs(S21));

% Display values in Command Window

fprintf('\n MICROWAVE TWO-PORT SIMULATION\n');

fprintf('-----------------------------------\n');

fprintf('S11 at 10 GHz             = %.3f dB\n',S11dB(i0));

fprintf('S21 at 10 GHz             = %.3f dB\n',S21dB(i0));

fprintf('Return Loss               = %.3f dB\n',RL(i0));

fprintf('Insertion Loss            = %.3f dB\n',IL(i0));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(i0));

fprintf('Best Operating Frequency  = %.2f GHz\n',f(ib));

fprintf('Maximum S21 Frequency     = %.2f GHz\n',f(it));

fprintf('\n VALUES AT BEST FREQUENCY\n');

fprintf('S11                       = %.3f dB\n',S11dB(ib));

fprintf('S21                       = %.3f dB\n',S21dB(ib));

fprintf('Return Loss               = %.3f dB\n',RL(ib));

fprintf('Insertion Loss            = %.3f dB\n',IL(ib));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(ib));

% Plot all six parameters

figure('Color','w');

subplot(2,3,1);

plot(f,S11dB,'b','LineWidth',1.5);

title('S11'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,2);

plot(f,S21dB,'r','LineWidth',1.5);

title('S21'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,3);

plot(f,RL,'m','LineWidth',1.5);

title('Return Loss'); xlabel('Frequency (GHz)');

ylabel('RL (dB)'); grid on;

subplot(2,3,4);

plot(f,IL,'k','LineWidth',1.5);

title('Insertion Loss'); xlabel('Frequency (GHz)');

ylabel('IL (dB)'); grid on;

subplot(2,3,5);

plot(f,Eff,'g','LineWidth',1.5);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)'); grid on;

subplot(2,3,6);

plot(f,S11dB,'b','LineWidth',1.5); hold on;

plot(f,S21dB,'r','LineWidth',1.5);

xline(f(ib),'k--','Best frequency');

title('Optimum Frequency');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('S11','S21','Location','best');

grid on;

sgtitle('Microwave Two-Port Network (8-12 GHz)');clc; clear; close all;

% Input parameters

f = 8:0.05:12;              % Frequency in GHz

f0 = 10;                    % Center frequency

Z0 = 50;                    % Reference impedance (ohms)

L = 50e-3;                  % Length in metres

% Illustrative two-port model

S11dB = -25 - 15*(f-10).^2;

S21dB = -2 - 0.1*(f-10).^2;

% Linear S-parameter magnitudes

S11 = 10.^(S11dB/20);

S21 = 10.^(S21dB/20);

% Performance calculations

RL = -20*log10(abs(S11));   % Return loss (dB)

IL = -20*log10(abs(S21));   % Insertion loss (dB)

Eff = abs(S21).^2*100;      % Transmission efficiency (%)

% Find values at 10 GHz

[~,i0] = min(abs(f-f0));

% Find optimum frequency

[~,ib] = min(abs(S11));

[~,it] = max(abs(S21));

% Display values in Command Window

fprintf('\n MICROWAVE TWO-PORT SIMULATION\n');

fprintf('-----------------------------------\n');

fprintf('S11 at 10 GHz             = %.3f dB\n',S11dB(i0));

fprintf('S21 at 10 GHz             = %.3f dB\n',S21dB(i0));

fprintf('Return Loss               = %.3f dB\n',RL(i0));

fprintf('Insertion Loss            = %.3f dB\n',IL(i0));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(i0));

fprintf('Best Operating Frequency  = %.2f GHz\n',f(ib));

fprintf('Maximum S21 Frequency     = %.2f GHz\n',f(it));

fprintf('\n VALUES AT BEST FREQUENCY\n');

fprintf('S11                       = %.3f dB\n',S11dB(ib));

fprintf('S21                       = %.3f dB\n',S21dB(ib));

fprintf('Return Loss               = %.3f dB\n',RL(ib));

fprintf('Insertion Loss            = %.3f dB\n',IL(ib));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(ib));

% Plot all six parameters

figure('Color','w');

subplot(2,3,1);

plot(f,S11dB,'b','LineWidth',1.5);

title('S11'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,2);

plot(f,S21dB,'r','LineWidth',1.5);

title('S21'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,3);

plot(f,RL,'m','LineWidth',1.5);

title('Return Loss'); xlabel('Frequency (GHz)');

ylabel('RL (dB)'); grid on;

subplot(2,3,4);

plot(f,IL,'k','LineWidth',1.5);

title('Insertion Loss'); xlabel('Frequency (GHz)');

ylabel('IL (dB)'); grid on;

subplot(2,3,5);

plot(f,Eff,'g','LineWidth',1.5);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)'); grid on;

subplot(2,3,6);

plot(f,S11dB,'b','LineWidth',1.5); hold on;

plot(f,S21dB,'r','LineWidth',1.5);

xline(f(ib),'k--','Best frequency');

title('Optimum Frequency');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('S11','S21','Location','best');

grid on;

sgtitle('Microwave Two-Port Network (8-12 GHz)');clc; clear; close all;

% Input parameters

f = 8:0.05:12;              % Frequency in GHz

f0 = 10;                    % Center frequency

Z0 = 50;                    % Reference impedance (ohms)

L = 50e-3;                  % Length in metres

% Illustrative two-port model

S11dB = -25 - 15*(f-10).^2;

S21dB = -2 - 0.1*(f-10).^2;

% Linear S-parameter magnitudes

S11 = 10.^(S11dB/20);

S21 = 10.^(S21dB/20);

% Performance calculations

RL = -20*log10(abs(S11));   % Return loss (dB)

IL = -20*log10(abs(S21));   % Insertion loss (dB)

Eff = abs(S21).^2*100;      % Transmission efficiency (%)

% Find values at 10 GHz

[~,i0] = min(abs(f-f0));

% Find optimum frequency

[~,ib] = min(abs(S11));

[~,it] = max(abs(S21));

% Display values in Command Window

fprintf('\n MICROWAVE TWO-PORT SIMULATION\n');

fprintf('-----------------------------------\n');

fprintf('S11 at 10 GHz             = %.3f dB\n',S11dB(i0));

fprintf('S21 at 10 GHz             = %.3f dB\n',S21dB(i0));

fprintf('Return Loss               = %.3f dB\n',RL(i0));

fprintf('Insertion Loss            = %.3f dB\n',IL(i0));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(i0));

fprintf('Best Operating Frequency  = %.2f GHz\n',f(ib));

fprintf('Maximum S21 Frequency     = %.2f GHz\n',f(it));

fprintf('\n VALUES AT BEST FREQUENCY\n');

fprintf('S11                       = %.3f dB\n',S11dB(ib));

fprintf('S21                       = %.3f dB\n',S21dB(ib));

fprintf('Return Loss               = %.3f dB\n',RL(ib));

fprintf('Insertion Loss            = %.3f dB\n',IL(ib));

fprintf('Transmission Efficiency   = %.2f %%\n',Eff(ib));

% Plot all six parameters

figure('Color','w');

subplot(2,3,1);

plot(f,S11dB,'b','LineWidth',1.5);

title('S11'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,2);

plot(f,S21dB,'r','LineWidth',1.5);

title('S21'); xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)'); grid on;

subplot(2,3,3);

plot(f,RL,'m','LineWidth',1.5);

title('Return Loss'); xlabel('Frequency (GHz)');

ylabel('RL (dB)'); grid on;

subplot(2,3,4);

plot(f,IL,'k','LineWidth',1.5);

title('Insertion Loss'); xlabel('Frequency (GHz)');

ylabel('IL (dB)'); grid on;

subplot(2,3,5);

plot(f,Eff,'g','LineWidth',1.5);

title('Transmission Efficiency');

xlabel('Frequency (GHz)');

ylabel('Efficiency (%)'); grid on;

subplot(2,3,6);

plot(f,S11dB,'b','LineWidth',1.5); hold on;

plot(f,S21dB,'r','LineWidth',1.5);

xline(f(ib),'k--','Best frequency');

title('Optimum Frequency');

xlabel('Frequency (GHz)');

ylabel('Magnitude (dB)');

legend('S11','S21','Location','best');

grid on;

sgtitle('Microwave Two-Port Network (8-12 GHz)');

OUTPUT

## Result

The microwave transmission lines and two-port networks were successfully simulated using MATLAB, and their performance was analyzed using S-parameters, reflection, and transmission characteristics.